产品规格书

Specification

客户Customer：

适用Apply：   蓝牙设备 Bluetooth Device

类型Type： 气传导蓝牙运动耳机 Air Conduction Open-Ear Headset

型号Model:  OPENEAR Duet Lite

样品数量Sample quantity：

编拟Layout：                	日期Date：

审核Auditing：              	日期Date:

批准Authorize：                日期Date：__________________

客  户Customer

生效日期 Effective Date:

客户印章Customer seal：______________

修改记录Modification Records

| 修改版次 Modification Time | 修改内容 Descriptions | 发行日期 Issued Date | 审核 Approved By |
| --- | --- | --- | --- |
| V1.0 | 无 |  |  |

目录Contents

4

4、5

5

5、6、7

8

8

9

9

9、10

10

- 适用范围Scope of Application .................................

- 参数信息Specifications..............................................

- 产品功能Product Functions......................................

- 产品使用User Manual...............................................

- 产品尺寸重量Product Size Weight...........................

- 包装示意图Product Package.....................................

- PCBA尺寸PCBA Size...............................................

- 运输注意事项Transportation..................................

- 使用时警告及注意事项Warning.............................

- 产品证书Certificates...............................................

- 适用范围 Scope of Application

- 本规范描述了本公司生产的开放双耳式蓝牙耳机有关参数要求和检验规则，本规范参考国家GB8898-2011音频产品安全及GB13837-2012 声音总规范。

- This specification describes the relevant parameter requirements and inspection rules of the Open-Ear bluetooth headset produced by the company. This specification refers to the national GB8898-2011 audio product safety and GB13837-2012 sound general specification.

- 参数信息Specifications

| 序号 NO. | 项目 Item | 参数 Details |
| --- | --- | --- |
| 2.1 | 内部型号Internal Model Name | AS3-T |
| 2.2 | 对外型号External Model Name | OPENEAR Duet Lite |
| 2.3 | 产品类型Product Type | 气传导蓝牙运动耳机Open-Ear Air Conduction Headphones |
| 2.4 | 声道系统Track system | 双声道立体声Two-channel stereo |
| 2.5 | 控制方式Control Mode | 1个物理功能按键 1 physical function button |
| 2.6 | 有源无源Active Passive | 有源 Active |
| 2.7 | 蓝牙方案Bluetooth Solutions | AC 6963A |
| 2.8 | 蓝牙版本Bluetooth Version | Bluetooth®v5.3 |
| 2.9 | 蓝牙协议Bluetooth Protocol | • A2DP v1.3 • AVRCP v1.6 • HFP v1.7 • HID v1.1 • AVCTP v1.4 • AVDTP v1.3 • SPP v1.2 |
| 2.10 | 蓝牙工作频率Bluetooth Frequency | 2402~2480MHz |
| 2.11 | 接收灵敏度TYP | -81dBm |
| 2.12 | 蓝牙有效距离Bluetooth range | ≈10m |
| 2.13 | 最佳通话距离TBR | ≤10m |
| 2.14 | 耳机最大功率Max power | 50MW |
| 2.15 | 耳机功放类型 Power Amplifier Type | Type D |
| 2.16 | 音频信噪比S/N | 85dB |
| 2.17 | 频率响应Frequency Response | 20Hz-20KHz |
| 2.18 | 扬声器单元Speaker Type | 08*10mm 动圈喇叭dynamic speaker *20R |
| 2.19 | 麦克风MIC | 硅咪silicone mic，- 42db |
| 2.20 | 电池Battery | 内置锂聚合物电池Li-polymer Battery 3.7V/120mAH |
| 2.21 | 音乐时间Music Time | ≈4.5h（80%音量volume） |
| 2.23 | 通话时间Talk Time | ≈ 5h（80%音量volume） |
| 2.23 | 待机时间（连接蓝牙） Standby Time (BT connected) | 30H |
| 2.24 | 待机时间（关机状态） Standby Time (Power Off) | 300H |
| 2.25 | 充电输入标准Charging Input | TYPE-C DC-5V |
| 2.26 | 充电时间Charging Time | 1.5h |
| 2.27 | 电池循环寿命Battery Life | 300次（80%剩余容量） 300times (80% remaining capacity) |
| 2.28 | 防水等级 IP Rating | IPX4 |
| 2.29 | 温度/湿度范围 Temperature/humidity range | 产品在干燥通风的室内，应避免与腐蚀性物质接触，远离火源及热源。产品在储存过程中须每个月充电1次。工作温度：0～40℃, 存储温度：-20～60℃。工作湿度：0%～85%，存储湿度：0%～90% The product should be in a dry and ventilated room, avoid contact with corrosive substances, and keep away from fire and heat sources. The product must be charged once a month during storage. Working temperature: 0～40℃, storage temperature: -20～60℃. Working humidity: 0%～85%, storage humidity: 0%～90% |

- 产品功能Product Functions

| 项目 Item |  | 功能特点 Functions |
| --- | --- | --- |
| 3.1 | 蓝牙立体声音乐播放 Bluetooth Stereo Music Playback | 支持连接蓝牙设备并支持双声道立体声高保真音乐播放。 Supports connection to Bluetooth devices and supports dual-channel stereo high-fidelity music playback. |
| 3.2 | 蓝牙通话 Bluetooth Phone Call | 支持蓝牙连接高清通话。 Supports Bluetooth connection for high-definition calls. |
| 3.3 | 语音提示 Voice Prompts | 内置语音提示。 Built-in voice prompts. |
| 3.4 | 电量指示/报警 Battery Indicator/Reminder | 支持iOS系统耳机电量指示及电池低电报警功能。 Support iOS system battery indicator and low battery alarm function. |
| 3.5 | 双色白绿LED指示 Two-color white and green LED indicators | 绿灯指示充电状态，白灯指示充满电状态。 The green LED indicates the charging state, and the white LED indicates the fully charged state. |

- 产品使用 User Manual

| 功能 Function |  | 操作 Operations | 提示灯 LED | 提示音 Prompts |
| --- | --- | --- | --- | --- |
| 4.1 | 开机 Power on | 耳机处于关机状态，长按按键，保持3秒。 Under power-off mode, press and hold the button for 3 seconds. | LED白灯闪烁3下 White LED flashes 3 times | power on |
| 4.2 | 关机 Power off | 耳机处于开机状态，长按按键，保持3秒。 Under power-on mode, press and hold the button for 3 seconds. | LED绿灯常亮3下 green LED flashes 3 times | power off |
| 4.3 | 蓝牙连接 BT connection | 关机状态，长按按键3秒，开机进入蓝牙配对状态，设备搜索到OPENEAR Duet Lite，点击连接。 In the power-off state, press and hold the button for 3 seconds to enter the Bluetooth pairing state. Search for OPENEAR Duet Lite in bluetooth menu and click to connect. | LED绿白灯闪烁 LED green and white light flashing | pairing |
| 4.4 | 连接成功 Connected | 耳机成功连接至设备。 The headset is successfully connected to the device | LED灯灭 LED light off | connected |
| 4.5 | 自动关机 Auto power off | 开机后未连接或者断开蓝牙连接后，如无任何操作，超过5分钟将自动关机，连上手机自动关机功能取消。 If it is not connected or disconnected from the Bluetooth connection after powering on, if there is no operation, it will automatically shut down after 5 minutes, and the automatic shutdown function when connected to a mobile phone will be canceled. | / | / |
| 4.6 | 取消拨号 Cancel call | 在拨出电话对方未接听前，单击按键，可取消拨号。 Before the other party does not answer the outgoing call, click the button to cancel the dialing. | / | / |
| 4.7 | 接听电话 Answer call | 来电时单击按键一下可接听来电。 When there is an incoming call, click the button once to answer the incoming call. | / | / |
| 4.8 | 拒绝来电 Reject call | 来电时双击 按键,拒绝对方来电。 Double-click the button to reject the incoming call. | / | / |
| 4.9 | 挂断电话 Hang up call | 通话完成，单击按键可结束通话。 When the call is complete, click the button to end the call. | / | / |
| 4.10 | 启动语音助手 Activate Voice Command | 快速3击按键 可启动语音助手功能。 Quickly triple-click the button to activate the SIRI function. | / | / |
| 4.11 | 播放/暂停音乐 Play/pause music | 音乐暂停状态，单击按键，可继续播放音乐。 音乐播放状态，单击按键，可暂停音乐。 When the music is paused, click the button to continue playing the music. When the music is playing, click the button to pause the music. | / | / |
| 4.12 | 开机自动回连 Auto connect | 耳机自动回连最后连接的设备。 The headset automatically reconnects to the last connected device. | / | / |
| 4.13 | 从手机端重连 Re-connect with phone | 从手机端断开耳机连接后，可从手机端重新连接耳机。 After disconnecting the headset from the mobile phone, you can reconnect the headset from the mobile phone. | / | / |
| 4.14 | 超出范围掉线后回连 Re-connect after exceeding connection distance | 当蓝牙超出工作范围后，5分钟内重回工作区域自动重新连接(如不能连接，单击中间键可自动连接）。 When the Bluetooth is out of the working range, it will automatically reconnect when it returns to the working area within 5 minutes (if it cannot be connected, click the middle button to automatically connect). | / | / |
| 4.15 | 电量显示 Battery display | 连接设备后可看到耳机的电量显示。 After connecting the device, you can see the battery power display. | / | / |
| 4.16 | 低电提示 Low battery prompt | 当电池电量低于20%时（3.3V播报低电提示3.0V自动关机）。 When the battery power is lower than 20% (3.3V prompts low battery, 3.0V automatic shutdown). | 每隔3分钟绿灯闪烁3下。 Green light flashes 3 times every 3 minutes | 3分钟循环一次，提示音：battery low |
| 4.17 | 低电关机 Power off low battery | 当电池电量等于0%时自动关机。 Automatically shuts down when the battery level drops 0%. | / | / |
| 4.18 | 充电(电流) Charging (current) | 接入充电接口可给设备充电，最大充电电流约150mA（电池120mA，充电时间1.5h） Connect to the charging interface to charge the device, the maximum charging current is about 150mA (battery 120mA, charging time 1.5H) | 充电中绿灯常亮，充满电白灯亮。 The green light is always on during charging, and the white light is on when fully charged. | / |
| 4.19 | 充电复位 Charging reset | 接入充电线后，耳机将会关机充电并复位。 After connecting the charging cable, the headset turn off to charge and reset. | / | / |
| 4.20 | 恢复出厂设置 Restore factory setting | 关机状态下，长按按键12秒恢复出厂。 Under power-off mode, press and hold the button for 12 seconds to restore the factory. | / | / |

- 产品颜色尺寸重量Product Size Weight

| 项目Item |  | 参数Details |
| --- | --- | --- |
| 5.1 | 产品重量Product Weight | 16g±3g |
| 5.2 | 产品尺寸Product Size | 118*103*55mm |
| 5.3 | 产品外观Product Photo |  |
| 5.4 | 产品颜色Colors | 蓝色Blue |

- 包装示意图Product Package

| 项目Item |  | 参数Details |
| --- | --- | --- |
| 6.1 | 包装盒种类PKG Type |  |
| 6.2 | 包装盒尺寸PKG Size |  |
| 6.3 | 包装盒重量PKG Weight |  |
| 6.4 | 包装内配件Accessory |  |
| 6.5 | 每箱数量QTY/CTN |  |
| 6.6 | 外箱尺寸CTN Size |  |
| 6.7 | 每箱净重CTN N.W. |  |
| 6.8 | 每箱毛重CTN G.W. |  |

- PCBA尺寸PCBA Size

| 项目Item |  | - 参数Details |
| --- | --- | --- |
| - 7.1 | - PCBA主板尺寸Size | - 32mm*8.5mm*0.8mm |
| - 7.2 | - PCB-TOP |  |
| - 7.3 | - PCB-BOTTOM |  |

- 运输注意事项Transportation

产品在半荷电（50-90%电量状态）下包装成箱进行运输，在运输过程中应防止剧烈振动、冲击或挤压，防止日晒雨淋，可适用汽车、火车、轮船、飞机等普通交通工具运输。

The product is packed in a box for transportation under half-charged (50-90% power state). During transportation, it should be protected from severe vibration, impact or squeezing, and prevent sun and rain. It can be used for cars, trains, ships, airplanes, etc. Ordinary transportation.

- 使用时警告及注意事项Warning

1. 切勿使用具有磨蚀性的清洁剂清理本产品。

2. 切勿让本产品加热或靠近火源，如煮食炉。

3. 切勿让本产品长时间被阳光直接照射。

4. 切勿将本产品放置于火中，避免发生爆炸。

5. 切勿尝试拆开本产品做出任何修改，因为它并不包含任何维修组件。

6. 切勿在雨中使用本产品。

7. 如果长时期不打算使用本产品，请您将它储存在干燥的环境中，避免极端的温度湿度和灰尘。并每1个月至少为耳机充电一次以延长电池寿命。

1. Do not use abrasive cleaners to clean this product.

2. Do not allow this product to heat up or close to fire sources, such as cooking stoves.

3. Do not expose this product to direct sunlight for a long time.

4. Do not put this product in fire to avoid explosion.

5. Do not try to disassemble this product to make any modifications, because it does not contain any repair components.

6. Do not use this product in the rain.

7. If you do not plan to use this product for a long time, please store it in a dry environment to avoid extreme temperature, humidity and dust. And charge the headset at least once every 1 month to extend battery life.

- 产品证书Certificates